Sharp LJ64ZU49/50/51/52 640 X 480 EL 16 Gray Level

| <TBODY>Signal | Pin No. | PC530 Signal | J8 | J9 | Description |
| --- | --- | --- | --- | --- | --- |
| N/C | A1 |  |  |  |  |
| N/C | B1 |  |  |  |  |
| D11 | A2 | PNL5 | 11 | 11 | Data |
| D10 | B2 | PNL4 | 9 | 9 | Data |
| D13 | A3 | PNL7 | 15 | 15 | Data |
| D12 | B3 | PNL6 | 13 | 13 | Data |
| D01 | A4 | PNL1 | 3 | 3 | Data |
| D00 | B4 | PNL0 | 1 | 1 | Data |
| D03 | A5 | PNL3 | 7 | 7 | Data |
| D02 | B5 | PNL2 | 5 | 5 | Data |
| N/C | A6 |  |  |  |  |
| N/C | B6 |  |  |  |  |
| CKD | A7 | XCLK | 21 | 41 | Data Shift Clock |
| GND | B7 | GND | 2 | 4 | Ground |
| HD | A8 | VEE/DE,DE | 25 | 40 | Data Latch ( Hsync ) |
| GND | B8 | GND | 4 | 6 | Ground |
| VD | A9 | OFLM | 17 | 38 | Scan Start Up ( Vsync ) |
| GND | B9 | GND | 6 | 10 | Ground |
| GND | A10 | GND | 8 | 16 | Ground |
| GND | B10 | GND | 10 | 20 | Ground |
| VL# | A11 | +12FUSED | 24 | 12 | External +24VDC |
| VL# | B11 | +12FUSED | 26 | 14 | External +24VDC |
| VL | A12 | +5FUSED | 18 | 26 | +5V Logic Power |
| VL | B12 | +5FUSED | 18 | 26 | +5V Logic Power |
| VD* | A13 | +12FUSED | 24 | 12 | +12V Power |
| VD* | B13 | +12FUSED | 26 | 14 | +12V Power</TBODY> |

Jumper Settings: J5 2-3, J12 2-3, J16 2-3, J6 1-2, J7 1-2, J11 2-3

Note: J5 and J12 are not installed on the PC525.

# For LJ64ZU49 which requires external +24VDC. These pins are not connected on +12VDC versions. ( See J10 Power Connector description. )

* These pins are not connected on +24 VDC versions.