Panasonic MD480B640PG3 640 X 480 Plasma 16 Gray Levels

| <TBODY>Signal | Pin No. | PC530 Signal | J8 | J9 | Description |
| --- | --- | --- | --- | --- | --- |
| VSYNC | 1 | OFLM | 17 | 38 | Scan Start Up ( Vsync ) |
| GND | 2 | GND | 2 | 4 | Ground |
| HSYNC | 3 | OLP | 19 | 43 | Hsync |
| GND | 4 | GND | 2 | 4 | Ground |
| DATAO0 | 5 | PNL3 | 7 | 7 | Data |
| GND | 6 | GND | 4 | 6 | Ground |
| DATAO1 | 7 | PNL2 | 5 | 5 | Data |
| GND | 8 | GND | 4 | 6 | Ground |
| DATAO2 | 9 | PNL1 | 3 | 3 | Data |
| GND | 10 | GND | 6 | 10 | Ground |
| DATAO3 | 11 | PNL0 | 1 | 1 | Data |
| GND | 12 | GND | 6 | 10 | Ground |
| DATAE0 | 13 | PNL7 | 15 | 15 | Data |
| GND | 14 | GND | 8 | 16 | Ground |
| DATAE1 | 15 | PNL6 | 13 | 13 | Data |
| GND | 16 | GND | 8 | 16 | Ground |
| DATAE2 | 17 | PNL5 | 11 | 11 | Data |
| GND | 18 | GND | 10 | 20 | Ground |
| DATAE3 | 19 | PNL4 | 9 | 9 | Data |
| GND | 20 | GND | 10 | 20 | Ground |
| DSPTMG | 21 | VEE/DE,DE | 25 | 40 | Data Enable |
| GND | 22 | GND | 12 | 24 | Ground |
| BACKG | 23 | N/C |  |  |  |
| GND | 24 | GND | 12 | 24 | Ground |
| CLOCK | 25 | XCLK | 21 | 41 | Data Shift Clock |
| GND | 26 | GND | 14 | 30 | Ground |
| J2 Power Connector |  |  |  |  |  |  |  |  |  |  |
| +12V | 1 | +12FUSED | 24 | 12 | +12VDC Power |
| +12V | 2 | +12FUSED | 26 | 14 | +12VDC Power |
| GND | 3 | GND | 14 | 30 | Ground |
| GND | 4 | GND | 14 | 30 | Ground |
| +5V | 5 | +5FUSED | 18 | 26 | +5V Logic Power</TBODY> |

Jumper Settings: J5 2-3, J12 2-3, J16 2-3, J6 1-2, J7 1-2, J11 1-2