Planar EL640.480-A3 640 X 480 EL 16 Gray Levels

| <TBODY>J1 Signal Connector |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | Pin No. | PC530 Signal | J8 | J9 | Description |
| GND | 1 | GND | 2 | 4 | Ground |
| D0 | 2 | PNL0 | 1 | 1 | Data |
| GND | 3 | GND | 4 | 6 | Ground |
| D1 | 4 | PNL1 | 3 | 3 | Data |
| GND | 5 | GND | 6 | 10 | Ground |
| D2 | 6 | PNL2 | 5 | 5 | Data |
| N/C | 7 | N/C |  |  |  |
| D3 | 8 | PNL3 | 7 | 7 | Data |
| N/C | 9 | N/C |  |  |  |
| D4 | 10 | N/C |  |  |  |
| N/C | 11 | N/C |  |  |  |
| D5 | 12 | N/C |  |  |  |
| N/C | 13 | N/C |  |  |  |
| D6 | 14 | N/C |  |  |  |
| GND | 15 | N/C |  |  |  |
| D7 | 16 | N/C |  |  |  |
| GND | 17 | GND | 8 | 16 | Ground |
| VCLK | 18 | XCLK | 21 | 41 | Data Shift Clock |
| GND | 19 | GND | 10 | 20 | Ground |
| _BLANK | 20 | N/C |  |  |  |
| GND | 21 | GND | 12 | 24 | Ground |
| HS | 22 | VEE/DE,DE | 25 | 40 | Data Latch ( Hsync ) |
| N/C | 23 | N/C |  |  |  |
| VS | 24 | OFLM | 17 | 38 | Scan Start Up ( Vsync ) |
| N/C | 25 | N/C |  |  |  |
| SELFTEST | 26 | GND | 14 | 30 | Ground |
| COLMAP | 27 | N/C |  |  |  |
| ENABLE | 28 | N/C |  |  |  |
| RESERVED | 29 | N/C |  |  |  |
| _LPOW | 30 | N/C |  |  |  |
| N/C | 31 | N/C |  |  |  |
| N/C | 32 | N/C |  |  |  |
| RESERVED | 33 | N/C |  |  |  |
| RESERVED | 34 | N/C |  |  |  |
| J2 Power Input Connector |  |  |  |  |  |  |  |  |  |  |
| VCC2 | 1 | +12FUSED | 24 | 12 | +12VDC Power |
| GND | 2 | GND | 32 | 42 | Ground |
| GND | 3 | GND | 34 | 44 | Ground |
| VCC1 | 4 | +5FUSED | 18 | 26 | +5V Logic Power</TBODY> |

Jumper Settings: J5 2-3, J12 2-3, J16 2-3, J6 1-2, J7 1-2, J11 2-3

Notes: J5 and J12 are not installed on the PC525. 

EL640.480 Panel Jumper Settings:

- Set VCC1 Jumper to " EXT " .

- Set DCON Jumper to " 1-D " .

- Set the DEFMAP Jumper to " BIN "

- Remove " VERPOS " jumpers as required for proper vertical centering.