Sanyo LM-CK53-22NEZ Single Scan Color LCD ( LCM 5330 )

| <TBODY>CN1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | Pin No. | PC530 Signal | J8 | J9 | Description |
| FLM | 30 | OFLM | 17 | 38 | Scan Start Up ( Vsync ) |
| M | 29 | OACDCLK | 22 | 36 | AC Frame Signal |
| DISP | 28 | +5SW | 16 | 28 | Display Control |
| CL1 | 27 | OLP | 19 | 43 | Data Latch ( Hsync ) |
| VSS | 26 | GND | 14 | 24 | Logic Gnd |
| CL2 | 25 | XCLK | 21 | 41 | Data Shift Clock |
| VSS | 24 | GND | 12 | 20 | Logic Gnd |
| UD0 | 23 | PNL6 | 13 | 13 | Data Upper |
| UD1 | 22 | PNL4 | 9 | 9 | Data Upper |
| UD2 | 21 | PNL2 | 5 | 5 | Data Upper |
| UD3 | 20 | PNL0 | 1 | 1 | Data Upper |
| UD4 | 19 | PNL14 | 41 | 29 | Data Upper |
| UD5 | 18 | PNL12 | 37 | 25 | Data Upper |
| UD6 | 17 | PNL10 | 33 | 21 | Data Upper |
| UD7 | 16 | PNL8 | 29 | 17 | Data Upper |
| LD0 | 15 | PNL7 | 15 | 15 | Data Lower |
| LD1 | 14 | PNL5 | 11 | 11 | Data Lower |
| LD2 | 13 | PNL3 | 7 | 7 | Data Lower |
| LD3 | 12 | PNL1 | 3 | 3 | Data Lower |
| LD4 | 11 | PNL15 | 43 | 31 | Data Lower |
| LD5 | 10 | PNL13 | 39 | 27 | Data Lower |
| LD6 | 9 | PNL11 | 35 | 23 | Data Lower |
| LD7 | 8 | PNL9 | 31 | 19 | Data Lower |
| VDD | 7 | +5FUSED | 18 | 26 | +5V Logic Power |
| VSS | 6 | GND | 10 | 16 | Logic Gnd |
| VSS | 5 | GND | 8 | 30 | Logic Gnd |
| VEE | 4 | +VLCD | 46 | 22 | + LCD Bias |
| VEE | 3 | +VLCD | 46 | 22 | + LCD Bias |
| VO | 2 | N/C |  |  | Wiper of Contrast Adj. Pot</TBODY> |

Jumper Settings: J5 1-2, J12 2-3, J16 2-3, J6 1-2, J7 2-3, J11 1-2

VO ( pin 2 of the display ) is normally connected to the wiper of a 10K pot used for contrast adjustment. The top of the pot is connected to VEE ( pins 3 and 4 of the display ). The bottom end of the pot is connected through a 20K resistor to ground.